Date of Ice Break and Ice Freeze

Solid = observed (Lake 239, up to 1 week different than L227)

Dotted = modeled

Ice break predicted to occur later than observed (RMSE = 5.25 days)

Ice freeze predicted to occur earlier than observed (RMSE = 16.5 days)

Influential factors:

- Albedo of ice and snow (break, not freeze)

- Inflow temperature

- Inflow volume

- Air temperature

- Wind speed

- Lake temperature stratification

Temperature

Epilimnion and hypolimnion temperatures are well-predicted. Metalimnion temperatures are predicted to be lower than observed.

Influential factors:

- POM in the epilimnion blocks light  blocks heat exchange

Chlorophyll a (proxy for phytoplankton biomass)

Highest observed chl a concentrations are underestimated by the model.

Chl a concentrations < 50 mg/L are sometimes overestimated by the model.

Influential factors:

- Half saturation P concentration for growth (under P limitation)

- Maximum specific growth rate

- Loss rate

- Specific chl a to P transformation rate

- PAR saturation level for phytoplankton growth

- Settling velocity for chl a

Percent cyanobacteria (in a 2 phytoplankton group situation)

Cyanobacterial growth is predicted to be approximately the same every year. This indicates that P-limited growth is not enough to accurately predict cyanobacterial vs. green algae growth in this system. We may need to add N limitation ot the model.

TDP

In general, TDP is overestimated by the model, although spikes in modeled TDP occur during the time of year when sampling does not occur (i.e., under ice). There seems to be a relationship in which decreasing modeled TDP results in increased modeled Total P (i.e., a tradeoff)

Total P (DIP, DOP, PP)

Total P is underestimated by the model. Increasing the modeled total P concentration results in a decrease in the modeled TDP concentration.